Summer Training Report.
Retail technical network
Telecom Egypt – We

Trainer Name:
Hazem Hamdi Mahmoud Abdelqader

University:
NUB – Nahda University

Faculty:
Faculty of Engineering Communications and Electronics Engineering

Training Provider:
We, Telecom Egypt

Company Supervisor:
Mr. Aymen Hossam El-Din

Academic Supervisor:
Engineer: Paula Atef Naguib

Email:
h.hamdy0491@nub.edu.eg

Phone Number:
+20 106 238 6571

Report Submission Date:
24 /8/2025

Training place: Alharih Central in Fayoum

Content:

- ADSL, VDSL, Supervector Router

- DSL Technical Troubleshooting

- ONT, ONU FTTH Routers

- FTTH Technical Troubleshooting

- MIFI, Wingle, WEair Mobile Internet Devices & Troubleshooting

ADSL, VDSL, Supervector Router

| Feature | ADSL Router | VDSL Router | SuperVector Router |
| --- | --- | --- | --- |
| Full Name | Asymmetric Digital Subscriber Line | Very-high-speed Digital Subscriber Line | Super Vectoring DSL |
| Download Speed | Up to 15 Mbps | Up to 100 Mbps | Up to 300 Mbps |
| Upload Speed | Up to 1 Mbps | Up to 30 Mbps | Up to 100 Mbps |
| Technology | Older copper line technology | Improved DSL over short copper lines | Enhanced VDSL with vectoring |
| Line Quality Sensitivity | Very sensitive to line quality | Moderate | Improved noise cancellation |
| Distance Limitation | Performs poorly beyond 2 km | Perform well up to 1 km | Best performance under 500 meters |
| Latency | Higher latency | Lower latency | Very low latency |
| Use Case | Basic browsing, email | HD streaming, online gaming | 4K streaming, smart home, heavy usage |
| Price | Lowest | Moderate | Highest |

DSL Technical Troubleshooting

- Physical issue

- Logical issue

Physical issue

- Cutting in the wire

- Wrong Card

- Physical Instability

- Cutting in the wire:

  - Data Dowen

  - Voice Dowen

  - Data & voice Dowen

- Wrong Card:

  - Two lines in the same port.

- Physical Instability:

    - cutting or rusted cable

    - Problem with the router's hardware

Logical issue

Managed by NOC team

- IP Conflict

- DNS Error

- DHCP

- IP Conflict:

  - Two Device in the same IP

- DNS Error:

  - When the Domain Name System fails to resolve a website address to its corresponding IP address.

- DHCP (Dynamic Host Configuration Protocol):

  - When a device fails to obtain an IP address from the DHCP server, this usually results in no internet access and can be caused by server issues, network misconfigurations, or timeouts.

ONT, ONU FTTH Routers

| Feature | ONT (Optical Network Terminal) | ONU (Optical Network Unit) |
| --- | --- | --- |
| Function | Converts optical signal to electrical for home use | Similar function, often used in shared buildings |
| Location | Typically installed inside the customer’s home | May be placed outside or in shared hallway |
| Ports | Includes Ethernet, VoIP, and sometimes Wi-Fi | May have fewer ports or connect to a router |
| Power Supply | Requires power from customer premises | Usually powered in a shared cabinet |
| Integrated Router | Often includes built-in router/Wi-Fi | Usually requires separate router |
| Usage by WE | Common in individual homes with FTTH lines | Used in buildings with multiple users |
| Speed Support | Supports full fiber speeds (up to 1 Gbps+) | Also supports high fiber speeds |

FTTH Technical Troubleshooting.

- Physical Layer Checks.

  - Inspect Fiber & Connectors

    - Clean all SC/APC or LC connectors with isopropyl wipes or specialized cleaning tools.

    - Check for bends, kinks or breaks along the cable route.

  - Verify Optical Power Levels

  - Use an Optical Power Meter to confirm Rx (–28 to –8 dBm) and Tx levels per PON spec.

  - If power is below threshold ⇒ look for dirty connectors, excessive attenuation, or splitter fault.

- ONT/ONU Status

- LED Indicators

  - PON LED off or blinking ⇒ no sync with OLT (check fiber & power).

  - LOS (Loss of Signal) ON ⇒ fiber break or dirty connector.

- Re-provisioning

  - Log into ONT’s web GUI or ACS (TR-069) server, force a re-registration (Reboot/Re-bind).

  - Check SN/Password match the OLT’s VLAN/PON port config.

- DHCP & IP Connectivity

- IP Address Assignment

  - Confirm ONT successfully obtains IP from DHCP server.

  - If no IP ⇒ check DHCP pools, option-82 settings, and VLAN tagging.

- Ping & Traceroute

  - From ONT CLI or a PC behind it, ping the OLT’s default gateway and public DNS (e.g., 8.8.8.8).

MIFI, Wingle, WEair Mobile Internet Devices

| Feature | MiFi | Wingle | WE Air |
| --- | --- | --- | --- |
| Device Type | Portable Wi-Fi hotspot | USB modem with hotspot feature | Fixed wireless home router |
| Portability | High – battery powered | Medium – USB powered (PC/laptop needed) | Low – intended for fixed home/office use |
| Power Source | Rechargeable battery | USB port | AC adapter |
| Internet Source | 4G/3G SIM | 4G/3G SIM | 4G/4G+ SIM or outdoor antenna |
| Wi-Fi Sharing | supports up to 10–15 devices | supports up to 5–10 devices | supports 32+ devices |
| Speed Support | Up to 150 Mbps | Up to 150 Mbps | Up to 300 Mbps (depending on model) |
| Best For | On-the-go internet (travel, outdoors) | Laptop/PC users needing occasional internet | Home/office users needing stable high-speed |
| SIM Type | Micro/Nano SIM | Standard/Micro SIM | Micro SIM (may vary) |
| Additional Features | Portable, LCD screen, battery status | Plug & play, compact size | LAN ports, stronger signal, better coverage |
| Price Range | Moderate | Budget-friendly | Higher – but more powerful |

MIFI, Wingle, WEair Mobile Internet Troubleshooting

No Internet Connection

- Check SIM Status
Ensure the SIM card is inserted properly and is active (not expired or blocked).

- Signal Strength
Confirm sufficient 4G/3G signal bars. Try moving to an open area or near a window.

- Data Balance
Use the WE app or USSD code to check if there’s remaining data quota.

- Restart Device
Power off the MiFi/Wingle/WE Air for 10 seconds, then turn it back on.

Device Not Detected (Wingle)

- USB Port Issues
Try a different USB port or another computer.

- Driver Missing
Install required drivers from the device’s virtual CD-ROM or official WE website.

- No Light Indicator
If LED doesn’t turn on, check power supply or device damage.

Slow Internet Speed

- Network Congestion
Test at a different time or location.

- Connected Devices
Too many users can slow performance—limit the number of devices connected.

- Signal Quality
Use the device web interface to check signal dBm level (ideal: –70 dBm to –90 dBm).

- Frequency Band Locking
In some advanced models, lock to 4G only or preferred band via settings (1800 MHz).

Wi-Fi Issues

- SSID Not Visible
Restart the device and check if Wi-Fi is turned on in the settings.

- Wrong Password
Reset password from the admin panel (usually 192.168.8.1 or printed on the back).

- Interference
Change the Wi-Fi channel to reduce interference from nearby networks.

WE Air – Specific Issues

- Poor Indoor Coverage
Use external antennas (if supported), or move the device near a window.

- SIM Not Recognized
Confirm SIM is data-enabled and supports LTE. Reinsert or try another SIM for testing.

ADSL, VDSL, Supervector Router Configuration.

ADSL Router – Huawei HG531 V1

- Connect to the router:

  - Connect your PC via LAN cable or Wi-Fi (SSID and password printed on label).

  - Open browser → enter IP: 192.168.1.1.

- Login:

  - Username: admin

  - Password: admin or found on router label.

- Setup ADSL Internet:

  - Go to Basic → WAN.

  - Set:

    - Username/Password: (provided by WE)

    - VPI/VCI: 0 / 35 (default for Egypt)

- Save & reboot.

- Wi-Fi Settings (optional):

  - Go to WLAN → Basic

  - Change SSID, password (WPA2 recommended).

VDSL Router – ZTE ZXHN H168N

- Connect via LAN or Wi-Fi.

  - IP: 192.168.1.1

- Login:

  - Username: admin

  - Password: On sticker (or admin)

- Setup VDSL WAN:

  - Go to Network → WAN

  - Choose WAN Type: VDSL

  - Enter WE username & password

- Save → Reboot.

- Change Wi-Fi Settings under Wireless → SSID Settings.

Supervector Router – Huawei DN8245V

Connect to router via LAN or Wi-Fi.

  - Default IP: 192.168.1.1.

- Login:

  - Username: telecomadmin

  - Password: On sticker

- Set WAN Connection:

  - Go to Internet → WAN

  - Input WE username/password

  - Apply settings

- Wi-Fi Setup:

  - Go to WLAN → SSID Settings

  - Set:

    - 2.4GHz and 5GHz names separately.

MIFI, Wingle, WEair Configuration.

MiFi Router – Huawei E5577Cs-321

- Connect to Wi-Fi:

  - SSID and Wi-Fi password are printed on the back or shown on the screen.

- Access Web Interface:

  - Open your browser → go to 192.168.8.1

  - Username: admin

  - Password: admin (or printed on the device)

- Internet Settings:

  - Navigate to Settings → Dial-up → Profile Management

    - Profile Name: WE

    - APN: internet.te.eg

  - Click Save, then Apply.

- Wi-Fi Settings (Optional):

  - Go to Wi-Fi Settings to change SSID and password.

- Restart the MiFi after applying changes.

Wingle Router – Huawei E8372h-608

- Plug the Wingle:

  - Insert into a laptop, car charger, or power bank USB port.

- Connect via Wi-Fi:

  - SSID and password are written on the back label.

- Access Web Interface:

  - Open browser → go to 192.168.1.1

  - Username: admin

  - Password: admin

- Set APN:

  - Go to Settings → Dial-up → Profile Management

    - Profile Name: WE

    - APN: internet.te.eg

  - Save → Apply.

- Wi-Fi Configuration:

  - Navigate to WLAN Settings to rename the SSID and change the password.

WE Air Router – ZTE MF253V

- Insert SIM & Power On:

  - Insert the 4G SIM card into the slot, then plug in and power on the router.

- Connect to the Router:

  - Use the default SSID and password (printed on the device) or LAN cable.

- Access Admin Interface:

  - Open browser → enter 192.168.1.1

  - Username: admin

  - Password: admin (or as printed on the label)

- Configure APN:

  - Go to Settings → Network → APN Settings

    - APN: internet.te.eg

    - Mode: Auto

  - Save and Apply.

- Wi-Fi Configuration:

  - Navigate to Wireless Settings → change SSID and password as needed.

- Optimize Signal:

  - Place the router near a window or higher position for better 4G signal strength.

ONT, ONU FTTH Routers Configuration.

ONT Router – Huawei EchoLife HG8245H

- Connect to the ONT:

  - Connect your PC to the ONT via LAN cable or Wi-Fi.

  - Open your browser and go to: 192.168.100.1

- Login:

  - Username: telecomadmin

  - Password: Usually on the sticker (admintelecom or admin@Huawei123)

- Internet Setup (PPPoE):

  - Go to WAN → WAN Configuration

  - Set:

    - WAN Mode: Route WAN

    - Connection Type: PPPoE

    - VLAN: Enable → ID: 835

    - Enter username & password provided by WE (user@te.eg)

- Wi-Fi Settings:

  - Go to WLAN → 2.4GHz / 5GHz

  - Change SSID (network name) and Wi-Fi password

  - Use WPA2-PSK or WPA3 if available for best security

- Save & Reboot

ONU Device – ZTE ZXHN F660 (used as ONU)

Typically used in apartment buildings or multi-tenant fiber connections, where it acts as a bridge between fiber and a home router.

- Connect to the ONU:

  - Usually connected to a router via Ethernet.

  - If accessible directly, connect via LAN and access 192.168.1.1

- Login:

  - Username: admin

  - Password: admin or from label

- Bridge Mode Configuration:

  - Go to Network → WAN → WAN Connection

  - Set:

    - Connection Type: Bridge

    - VLAN ID: 835

    - Save settings

- Use External Router (TP-Link, Huawei):

  - Connect ONU LAN port to WAN port of the external router

  - Configure PPPoE on the router with WE credential

  - Set up Wi-Fi, DHCP, and NAT on the external router

- Save & Reboot Both Devices

important terms.

- CPE : Customer Premises Equipment.

- BLQ : Bad Line Quilty.

- STB : Receiver which responsible for IPTV.

- DHCP: Dynamic Host Configuration Protocol.